a2A2840-14gmUV

The a2A2840-14gmUV Basler ace 2 X camera is equipped with a Sony IMX487 sensor.

This ultraviolet (UV) camera with a GigE interface can capture images in the invisible light spectrum with wavelengths from 200–400 nm, the most suitable UV range for industrial inspections.

Specifications

General Specifications

|  | a2A2840-14gmUV |
| --- | --- |
| Resolution (H x V Pixels) | 2856 x 2848 (full resolution) 2840 x 2840 (default resolution) |
| Resolution | 8 MP |
| Sensor Type | Sony IMX487-AAMJ-C Progressive scan CMOS Global shutter |
| Sensor Format | 2/3" |
| Effective Sensor Diagonal | 11.1 mm |
| Pixel Size (H x V) | 2.74 x 2.74 µm |
| Frame Rate (at Default Settings) | 12.7 fps 14.5 fps (with Bandwidth Reserve mode set to Performance and packet size set to 4000) |
| Product Family | ace 2 X |
| Mono / Color | Mono |
| Image Data Interface | Fast Ethernet (100 Mbit/s) Gigabit Ethernet (1000 Mbit/s) |
| Pixel Formats | See Pixel Format. |
| Synchronization | Via hardware trigger Via software trigger Via free run |
| Exposure Time Control | Via hardware trigger Programmable via the camera API |
| Camera Power Requirements | ≈4.6 W (typical) when using Power over Ethernet ≈3.4 W (typical) @ 12–24 VDC when supplied via I/O connector |
| I/O Lines | 1 opto-coupled input line 2 general purpose I/O (GPIO) lines |
| Lens Mount | C-mount |
| Size (L x W x H) | 48.9 x 29 x 29 mm (without lens mount or connectors) 62.2 x 29 x 29 mm (with lens mount and connectors) |
| Weight | <105 g |
| Conformity | CE (includes RoHS), EAC, UL (in preparation), FCC, KC, GenICam, GigE Vision, IP30, IEEE 802.3af (PoE) Certificates for your camera model For more information, see the Compliance section of the Basler website. |
| Software | Basler pylon Software Suite (version 6.0 or higher) Available for Windows, Linux x86, Linux ARM, macOS, and Android |

Accessories -

Lenses –

2200002148	AZURE-00202XZMUV-1 	AZURE-00202XZMUV-1 UV Lens 25.0mm FL

2200002149 	AZURE-00302XZMUV-1	AZURE-00302XZMUV-1 UV Lens 35mm FL

2200002150	AZURE-00503XZMUV-1	AZURE-00503XZMUV-1 UV Lens 50mmFL

2200002151	AZURE-00705XZMUV-1	AZURE-00705XZMUV-1 UV Lens 75mm FL

Cables –

USB Cable - with screwlock	2200000620

USB Cable - with screwlock	2200000621

USB Cable - without screwlock 2000035131

USB Cable - without screwlock 2000035315

Drawing –

Please contact OEM Automatic for technical assistance.

Email: vision@oem.co.uk

Telephone: 01162849900